pages

106, 120

Energy transfer and efficiency

Extension

Sheet

An Energy Transfer Diagram (Sankey diagram) 
shows what happens during an energy transfer :

Efficiency=useful energy transferredtotal energy input×100%

energy lighting

the room

(useful energy)

#### Questions

For each question show all your working clearly.

- A lot of energy is wasted in a car. For every
100 J of chemical energy in the petrol, only
25 J are transferred to useful kinetic energy.
The rest just heats up the engine and the air.

- Draw an Energy Transfer Diagram for this,
to scale.

- Calculate the efficiency.

- The diagram shows the energy transfers for a
Bunsen burner heating a beaker of water.
What is its efficiency as a water heater?

- In a solar cell, for every 80 J of solar energy
shining on it, only 4 J is transferred to useful
energy (as electricity).

- What happens to the other 76 J?

- What is its efficiency?

- Draw a Sankey diagram of this, to scale.

- A pulley system lifts a load and gives it
6000 J of potential energy. The person
pulling on the rope gives it 8000 J of energy.
What is the efficiency?

- An electric kettle has a power rating of 2 kW
and is switched on for 100 seconds.
While heating up, it loses 60 000 J to the
surroundings.

- How much energy is supplied to the kettle?
(1 kW = 1000 W = 1000 joules per second)

- How much is given to the water?

- What is the efficiency of heating water?

- An electric motor on a building site has a
power rating of 400 W and lifts a load of
bricks weighing 600 N through a height of
10 m in 20 seconds.

- How much energy is needed to lift the bricks?
(See page 101 or page 112.)

- How much energy is supplied to the motor
in 20 seconds?

- What is the efficiency of the motor in
doing this job?