Investigating acceleration

Help

Sheet

pages
126, 133

How does the acceleration of a car

depend on the steepness of the slope?

The card is shaped as shown so that it cuts

the beam of light (in the light-gate) twice.

The 2 parts of the card are the same length.

While the card is cutting the beam of light,

the timer is running in the computer.

If the car is accelerating, the second part of

the card will cut the beam of light for a shorter time

(because it is now travelling faster).

From this, the computer can calculate the acceleration.

(You need to tell the computer the length of the card interrupts.)

Predict what you think will happen as you change

the steepness of the slope, or the weight of the car.

Test your predictions by experiments.

How does the computer calculate the acceleration?

Suppose that while the first part (A) is cutting the
beam of light, the timer counts 0.2 seconds.
Then (from page 126) :

Suppose a similar calculation for B gives 80 cm/s

Now suppose the time taken to travel from the middle of A to
the middle of B is measured by the timer as 0.3 seconds.

Then (from page 126) :

Acceleration =change in velocitytime taken for change=80−500.3=300.3=100 cm/s2  (1 m/s2)

Average
speed

= distance travelledtime taken=10 cm0.2 s=50 cm/s