page

127

Extension

Sheet

Investigating speed – 2

What happens to the speed of a trolley as it rolls down a hill ?

- You can find the speed of the trolley from the
time taken for the card to pass through the
light-gate (as described below).

- Predict what you think will happen if you:
-	change the steepness of the slope
-	add weights to the trolley.

- Test your predictions by experiments.

- Explain your results in terms of energy,
gravity and friction.

- Is there a connection between the speed of
the trolley and the vertical height it falls?

#### Finding the speed of the trolley

The ‘light-gate’ has a lamp shining across the
gap on to a light detector.
(This detector can be an LDR, see page 325.)

The light-gate is connected to an electronic timer.
When the card on the trolley passes through the
light beam, it stops the light from shining on
the detector. This starts the electronic timer.

When the card has passed through, the light
shines on the detector again and the timer stops.

If the card is 10 cm long, the timer tells you the
time for the card to travel 10 cm.
From this you can calculate the speed of the trolley.

average

speed

=

##### Example

The time for a 10 cm card is

0.2 seconds.

What is the speed?

Answer

=

distance travelledtime taken=10 cm0.2 s=50 cm/s