Revision

QUIZ

32Heating effect of a current

Use these questions to help you revise (cover the right-hand side with a sheet of paper).

| 1 | What are the energy transfers in a hair-dryer? | ... | energy in the power station is transferred by electricity to: - kinetic energy (of the motor and the air) - heat (of the heating coils and the air) - sound. |
| --- | --- | --- | --- |
| 2 | In a mains circuit, where are the switches and fuses placed? (pages 271, 275) | ... | always in the live wire. |
| 3 | Explain the difference between d.c. and a.c. (p. 274, 305) | ... | d.c. = direct current, like that from a battery. It flows one way. a.c. = alternating current, as in the mains. It is constantly changing direction, at a frequency of 50 Hz. |
| 4 | If the energy from the mains costs 12p per Unit (kW h), what is the cost of 20 Units (20 kW h)? (p. 273) | ... | Cost = number of kW h  cost per kW h = 20  12 = 240p (£2.40) |
| 5 | How should the wires be connected in a mains plug? (p. 276) | ... | - brown wire to the fuse (and so to the live pin) - blue wire to the neutral pin - green/yellow to the earth pin. |
| 6 | What is a watt? (p. 272, 114) | ... | a watt is a unit of power, or rate of energy transfer. 1 watt = 1 joule per second. 1 W = 1 J/s |
| 7 | A 230 V mains supply delivers a current of 2 A. What is the power? (p. 272) | ... | Power, P = p.d., V  current, I = 230 V  2 A = 460 W |
| 8 | What fuse would you use in this case (question 7)? (p. 276) | ... | the current is 2 A, so the next available standard fuse rating is 3 A. |
| 9 | In question 7, how much energy is transferred in 2 minutes? (p. 272) | ... | energy transferred = power, P  time, t = 460 W  120 s = 55 200 joules |
| 10 | A 2 kW electric fire is switched on for 10 hours. How much energy is transferred? (p. 273) | ... | energy transferred = power  time = 2 kW  10 hours = 20 kW h |