35Magnetic effect of a current

Checklist



# Name	Class

While studying this topic, tick off each item after you have covered it.

What I should know:

When a current flows through a wire,

it creates a magnetic field.

The magnetic field round a straight wire is

circular (cylindrical in 3D).

When a current flows through a coil (or a

solenoid) it acts like a bar magnet.

It is an electromagnet.

The shape of the magnetic field is like that

of a bar magnet.

If the current in an electromagnet is

reversed, the poles are reversed.

The strength of an electromagnet can be increased:

by using a larger current,

by increasing the number of turns on the

coil,

by using an iron core.

Electromagnets are used in:

cranes in steelworks and scrapyards

electric bells (page 294)

electric door-locks (page 294)

circuit-breakers (pages 278)

relays (pages 295, 327)

telephone earpieces (page 295)

loudspeakers (page 297)

When a wire carrying a current is placed

across a magnetic field, there is a force on it.

This is the motor effect.

The force can be increased:

by using a larger current,

by using a stronger magnetic field,

by having a longer length of wire in the field.

Force F = flux density, B × current, I × length, l

(newtons)         (tesla)                  (amperes)      (metres)

The direction of the force is reversed if

either   the current is reversed,

orthe magnetic field is reversed.

This effect is used in:

loudspeakers,

electric motors,

moving-coil ammeters and voltmeters.

What I should be able to do:

Recognise that the field round an

electromagnet is like that of a bar magnet.

Investigate to find out what affects the

strength of an electromagnet.

Describe how to make a strong

electromagnet.

Explain how an electric bell works.

Explain how a relay works.

Explain how a circuit-breaker works.

Use the formula:   F = B × I × l

in calculations.

Explain how a loudspeaker works.

Use Fleming’s Left-Hand Rule.

Understand how an electric motor is built,

and explain how it works.