A current in a wire makes a magnetic field.

The field round a straight wire is circular.

Electromagnets are used in:

•	cranes	•	electric bells

•	circuit-breakers	•	relays

The field round a coil is like a bar magnet.

If the current is reversed, the poles are reversed.

The strength of an electromagnet is increased by:

•	using a larger current

•	using more turns on the coil

•	using a ‘soft’ iron core.

When a wire carrying a current is placed across a

magnetic field, there is a force on it.

This is used in: •	electric motors • loudspeakers

The force F can be increased by:

•	using a larger current I

•	using a stronger magnetic field B

•	having a longer length l of wire in the field.

F  =  B  ×  I   ×  l

N

S

35   Magnetic effect of a current