35Magnetic effect of a current

Revision

QUIZ

Use these questions to help you revise  (cover the right-hand side with a sheet of paper).

| 1 | What happens when a current is passed through a straight wire? (page 292) | ... | it produces a magnetic field. The shape of the field is circular round the wire. |
| --- | --- | --- | --- |
| 2 | What happens when a current is passed through a coil (or a solenoid)? (p. 293) | ... | it produces a magnetic field like a bar magnet. This is an electromagnet. |
| 3 | What happens if you reverse the current in an electromagnet? (p. 293) | ... | the poles (and the field) are reversed. |
| 4 | How could you increase the strength of an electromagnet? (p. 293) | ... | By using: - a larger current, - more turns on the coil, - an iron core. |
| 5 | What is a relay used for? (p. 295) | ... | so that a small current in one circuit can switch on or off a big current in another circuit. |
| 6 | If an alternating current with a frequency of 1000 Hz is passed through the coil of a loudspeaker, what happens? (p. 297) | ... | because of electromagnetism, the coil will vibrate at a frequency of 1000 Hz. This makes the cone vibrate at 1000 Hz, and so produce a sound wave at 1000 Hz, which we can hear. |
| 7 | What happens to a wire carrying a current that is placed in a magnetic field? (p. 296) | ... | a force is exerted on the wire (providing the wire is positioned across the field). This is the motor effect. |
| 8 | How can the force on this wire be reversed in direction? (p. 296) | ... | either reverse the current, or reverse the magnetic field. |
| 9 | How can the force on the wire be made bigger? (p. 296) | ... | by using: - a larger current, - a stronger magnetic field, - a longer wire. |
| 10 | A wire is 0.30 m long. It carries a current of 5.0 A and is placed at right angles to a uniform field of magnetic flux density 0.16 T (tesla). What is the force on the wire? (p. 297) | ... | Force F = flux density, B × current, I × length, l = 0.16 T × 5.0 A × 0.30 m = 0.24 N |
| 11 | Use Fleming’s Left-Hand Rule to state which way this wire will move. (p. 296) | ... | out of the paper. |