page

298

Extension

Sheet

Moving-coil ammeters, voltmeters

A moving-coil meter is built like a motor, but

without a commutator.

(See Experiment 35.7 on page 298.)

When a current passes through the coil, the

magnetic field turns the coil (because of the motor effect, page 296).

The bigger the current, the bigger the turning

force.

The moving-coil meter has :

a)Two coil springs to return the pointer to zero

when the current stops.

These springs also conduct the current in and

out of the coil.

b)The magnet has curved pole faces and there

is a soft-iron core fixed at the centre of the

moving coil.

This means that the coil is always in line with

a strong uniform magnetic field. This ensures

that the meter has the advantage of a linear,

evenly-divided scale.

Disadvantages of this meter: a moving-coil meter
must be connected the correct way round in a
circuit (so the pointer moves the right way).
It cannot be used with alternating current (unless
a rectifier is used – see page 323).

This moving-coil meter or galvanometer can
be made so sensitive that it can measure a
micro-amp or even less.

Ammeter

A big current would damage the galvanometer.

To prevent this, a ‘shunt’ resistor with a very

low resistance is placed in parallel across the

galvanometer.  This takes most of the current,

rather like a motorway by-pass will take most of

the traffic round a village instead of through it.

Voltmeter

A voltmeter also uses a galvanometer, but this

time a high resistance is placed in series with

the meter, so that only a small current can flow

through it.  The larger the voltage, the larger the

current and the more the pointer moves.