Checklist



38Electronics

# Name		Class

While studying this topic, tick off each item after you have covered it.

What I should know:

A diode is made from semi-conductor

material.

It allows a current to flow one way only.

The arrow-head in the circuit symbol

shows the direction the current can flow.

Diodes have several uses.

A special kind of diode gives out light. It is

a light-emitting diode (LED).

A light-dependent resistor (LDR) changes

its resistance when light shines on it.

In bright light it has a low resistance.

In the dark it has a high resistance.

A thermistor changes its resistance when

the temperature changes.

When it is warm it has a low resistance.

When it is cold it has a high resistance.

A reed switch is a switch that is operated

by a magnetic field.

A relay uses a small current to switch on/

off a bigger current. (See also page 295.)

There are 3 main kinds of logic gate:

AND, OR, NOT.

Components such as LDRs and thermistors

can be used in a potential divider circuit,

to give a logical 0 or a logical 1.

The potential divider circuit can then be

connected to logic gates to make light-controlled circuits and

temperature-controlled circuits.

A variable resistor in the potential divider

circuit is used as a sensitivity control.

What I should be able to do:

Recognise and use the circuit symbol for

a diode.

Sketch a current–voltage graph for a

diode, and explain what the graph shows.

(See also page 265.)

Explain how a diode can be used to

rectify a.c.

Draw a simple circuit diagram with an

LDR and a cell in it, and explain what

happens to the current in daylight or dark.

Draw a simple circuit diagram with a

thermistor and a cell in it, and explain

what happens to the current when it is

hot or cold.

Explain how a relay works.

Explain why a relay coil needs a diode

connected across it.

Remember the truth tables for AND, OR,

NOT gates.

Understand and explain system diagrams

using up to 3 logic gates.

Understand how a potential-divider circuit

is used to give a logical 0 or logical 1.